# Abstract

Detailed finite element models(DFEMS) are quickly becoming the new standard for all new development of aerospace structures. However, as refined as the DFEMs are becoming, they still tend to be highly refined loads models and not true stress models. This forces analysts to revert back to using simple kt charts from resources like Peterson. This is often overly conservative with geometry that is not truly reflective of the designed geometry. For a flat plate model containing compounding Kts, this paper will develop a python based finite element solver that will create a “break out” FEM model, re-meshing critical locations with refined 8 noded quadrilateral elements, while leaving the surrounding non-critical elements as 4 noded quadrilateral elements. To bridge the higher and lower order element mesh, 7 noded transition elements are developed and implemented. Freebody loads are then applied from the DFEM to get true stress concentration values that can be used for classical Linear Elastic Fracture Mechanics analysis of a corner cracked plate. 

Results of the refined mixed mesh plate will be compared against classical elasticity solutions to ensure validity. Additionally, results will be compared against traditional element refinement methods relying on a substantial increase in lower order elements to determine the performance differences between the two refinement methods.

# General Overview of the Finite Element Method

We have four parts of the structural problem.

- Kinematics

- Constitutive

- Equilibrium

- Boundary Conditions

The kinematic equations refers to the strain displacement relations, the constitutive equations are the stress-strain equations.

Stress vector is as follows:

σ=σxσyσzτxyτyzτzxT

Strain vector is as follows:

ϵ=ϵxϵyϵzγxyγyzγzxT

The constitutive equations to relate strain to stress can be done with Hooke’s law assuming either plane stress or plain strain with matrix [E].

σ=Eϵ

The plane strain relation is defined as the following for the 2 dimensional assumption

σxσyτxy=E1+ν1-2ν1-νν0-ν10001-2ν2ϵxϵyγxy

Or for Plane stress the following can be used for the 2d assumption

σxσyτxy=E1-v21ν0ν10001-ν2ϵxϵyγxy

From this we can use the principal of virtual work to define the kinematic equations.

## Deriving The Statement of Potential Energy

For a liner-elastic body the total potential energy assuming small strain

Πu=Uu-Wextu

Where U is the strain energy and W is the work done by external forces and u is the displacement field

The internal potential energy is defined as Strain energy which is the work done by the internal stresses while deforming a body from zero strain to its final strain state

dU=σTdϵ

Need to integrate over the entire deformation path:

U=V0ϵσTdϵ

Stress is related to strain by the constitutive matrix σ=Cϵ

U=V0ϵCϵTdϵ

Integrating with respect to epsilon yields

Uu=12VϵTCϵdV

External Work

Assuming there are no body forces or surface tractions

Wextu=iuiTPi

This gives the potential energy statement of:

Πu=12VϵTCϵdV-iuiTPi

Knowing that the virtual work should be equal to zero we can take the first variation of the potential energy statement

δΠu=0

Taking the first variation of the potential energy statement

δΠ=δU-δWext=VϵTCδϵdV-iδuiTPi=0

Gives

VϵTCδϵdV=iδuiTPi

## Potential Energy / Virtual Work

One strategy for solving the stiffness matrix of an element is to employ the concept of virtual work and minimum potential energy. The expression for the potential energy of  a linear elastic body is as follows:

Πρ=v12ϵTEe-ϵTEϵ0+ϵTσ0dV- vuTFdV-suTΦdS-DTP

It will be assumed that there are no initial strains or stresses, no body forces, and no surface tractions. Therefore, the expression of potential energy can be reduced to:

Πρ=V(12{ϵ}TEϵ)dV-DTP

Where ϵ, u are defined as

u=N{d} Where N is the matrix of shape functions

ϵ=∂u Strain is known to be the partial derivatives of displacement

{d} is the elemental degrees of freedom ie {u1v1u2v2}

There fore substituting u into the epsilon gives

ϵ=∂N{d}

Next, we will define the B matrix as the partial derivatives of the shape functions

B=∂[N] therefore substituting into the strain displacement relationship gives

ϵ=B{d}

Substituting into the potential energy statement

Πρ=V(BTEB-NF{F})dV

From this the elemental stiffness is seen as

ke=VeBTEBdV

And the load vector as

re=VNTFdV

# Stiffness Derivation for a Quad Element

The previously shown elemental stiffness calculation’s volume can be calculated across x, y for a quadrilateral element.

ke=∫∫BTEBt dxdy

Where B is a scaling matrix and C is the Constitutive matrix

Assumes that t is constant within the element

In order to account for non-square elements a mapping function into the ξ,η

Using a linear approximation of nodal displacements defined as

disp=uv= a0+a1x1+a2x2+a3x1x2b0+b1x1+b2x2+b3x1x2

Coordinates x, y can be defined as the following

xy=∑Nixi∑Niyi

This expands to

xy=N100N1    N200N2     N300N3    N400N4x1y1    x2y2    x3y3    x4y4T

Displacements can be defined as the following and expanded similarly to the above form

uv=∑Niui∑Nivi

However as the quadrilateral elements are never perfectly square, evaluating the integrals between x, and y bounds becomes very difficult. Thus it is preferred to create a natural coordinate system with the coordinates  ξ, η and mapping the global coordinates into the natural system with the elements Jacobian being used to account for the shape distortions.

Where N are the shape functions that can be used for a liner quadrilateral elements which are also used in the mapping from the global and natural coordinate systems

N1=141-ξ1-η,	N2=141+ξ1+η

N3=141+ξ1+η,	N4=141-ξ1+η

The problem encountered is that the shape functions are in term so of ξ, η and not u, v. Thus a coordinate transformation must take place.

To take this coordinate transformation we employ the Jacobian matrix

xiyi= J ξiηi

Where the Jacobian equals

J =∂x∂ξ∂x∂η∂y∂ξ∂y∂η

The bounds of the integration can now be written in-terms of ξ, η with the following relation

dxdy=det J dξdη

Therefore

ke(x, y)=t∫BTCBdet⁡[ J] dξdη

Element Strain Displacements are given by

ε=Bd

εxεyγxy=100000010110∂u∂x∂u∂y∂v∂x∂v∂y

To get results in terms of ξ, η

u,xu,yv,xv,y=ξ,xη,x00ξ,yη,y0000ξ,xη,x 00ξ,yη,yu,ξu,ηv,ξv,η

Substituting in the strain displacement relation

εxεyγxy=100000010110ξ,xη,x00ξ,yη,y0000ξ,xη,x 00ξ,yη,yu,ξu,ηv,ξv,η

Note the at the displacements are defined as

uv=N100N1    N200N2     N300N3    N400N4u1v1    u2v2    u3v3    u4v4T

As the shape functions are functions of ξ, η their derivatives can be taken

u,ξu,ηv,ξv,η=N1,ξ0N2,ξ0N3,ξ0N4,ξ0N1,η0N2,η0N3,η0N4,η00N1,ξ0N2,ξ0N3,ξ0N4,ξ0N1,η0N2,η0N3,η0N4,ηu1v1u2v2u3v3u4v4

Substituting in the final strain displacement relation is found by

εxεyγxy=100000010110ξ,xη,x00ξ,yη,y0000ξ,xη,x 00ξ,yη,yN1,ξ0N2,ξ0N3,ξ0N4,ξ0N1,η0N2,η0N3,η0N4,η00N1,ξ0N2,ξ0N3,ξ0N4,ξ0N1,η0N2,η0N3,η0N4,ηu1v1u2v2u3v3u4v4

With the B Matix being

B=100000010110ξ,xη,x00ξ,yη,y0000ξ,xη,x 00ξ,yη,yN1,ξ0N2,ξ0N3,ξ0N4,ξ0N1,η0N2,η0N3,η0N4,η00N1,ξ0N2,ξ0N3,ξ0N4,ξ0N1,η0N2,η0N3,η0N4,η

Where the second matrix can be alternatively shown by

B=100000010110 J-1T00000000 J-1TN1,ξ0N2,ξ0N3,ξ0N4,ξ0N1,η0N2,η0N3,η0N4,η00N1,ξ0N2,ξ0N3,ξ0N4,ξ0N1,η0N2,η0N3,η0N4,η

# Gauss Integration

Gause integration says that an integral can be evaluated as a summation of functions evaluated at integration points with weights applied