# Abstract

Detailed finite element models(DFEMS) are quickly becoming the new standard for all new development of aerospace structures. However, as refined as the DFEMs are becoming, they still tend to be highly refined loads models and not true stress models. This forces analysts to revert back to using simple kt charts from resources like Peterson. This is often overly conservative with geometry that is not truly reflective of the designed geometry. For a flat plate model containing compounding Kts, this paper will develop a python based finite element solver that will create a “break out” FEM model, re-meshing critical locations with refined 8 noded quadrilateral elements, while leaving the surrounding non-critical elements as 4 noded quadrilateral elements. To bridge the higher and lower order element mesh, 7 noded transition elements are developed and implemented. Freebody loads are then applied from the DFEM to get true stress concentration values that can be used for classical Linear Elastic Fracture Mechanics analysis of a corner cracked plate. 

Results of the refined mixed mesh plate will be compared against classical elasticity solutions to ensure validity. Additionally, results will be compared against traditional element refinement methods relying on a substantial increase in lower order elements to determine the performance differences between the two refinement methods.

# Problem Overview

Within the industry of Fatigue and Damage Tolerance (F&DT) engineering for aviation, Damage Tolerance (DT) analysis is done using commercially available Linear Elastic Fracture Mechanics (LEFM) like NASGROW or AFGROW. Figure 1 shows a typical through thickness crack edge crack for a finite width plate subject to tension load. In this solution, beta accounts for the changes in stress intensity as the crack grows along the width of the plate.

Figure 1: Example Through Thickness Edge Crack Finite Width Plate Solution [1]

However, in actual aerostructures, a pure finite width plate is never actually seen. Thus, it is necessary to make assumptions between actual geometry and the solution’s assumed geometry. For example, a representative vertical frame is shown in Figure 2, and Figure 3 shows the idealized structure that would match the solutions assumed geometry.

Figure 2: Representative Vertical Frame

Figure 3: Assumed Finite Width Through Thickness Edge Crack Specimen

However, with the actual geometry reducing plate widths with the runout into a fillet, a stress concentration will be present. The through thickness edge crack for a finite width plate has no means of capturing this stress concentration thus a Kt must be calculated and applied on top of the traditional solutions beta. This changes the stress intensity calculation to be as shown below.

K=σKtβ*πa

Traditionally this Kt value would be calculated from an industry standard handbook like Peterson’s Stress Concentrations [2]. However, again the models within [2] do not perfectly match the actual structure. Leading to analyst having to choose between possibly overly conservative vs possibly un-conservative Kt value.

Figure 4: Kt Comparisons

Typical AVFEM would mesh the part shown in Figure 3 to level of detail shown in Figure 5.

Figure 5: AVFEM Representation

With this subprime mesh shown and the limitations of LEFM solutions and Kt solutions explained, the goal of this paper is to develop a finite element solver within python that is capable of solving standard Nastran elements like: 3 node triangles, 4 node quadrilaterals, and 8 node quadrilaterals. Additionally, the solver will be capable of solving transition elements that are 6 or 7 node quadrilaterals. These transition elements bridge the gap between 4 node and 8 node quadrilaterals.

# Finite Element Method Derivation

The four parts of the structural problem.

- Constitutive

- Kinematics

- Equilibrium

- Boundary Conditions

## Constitutive Equations

The constitutive equations relate stress to strain using hookes law. Two assumptions can be made that either the element is in plane strain (ϵzz=0) or the that the element is in plane stress (σzz=0). Both relations are given below. As the assumed thickness throughout all plates, plane stress is used in all analysis.

Stress vector:

σ3d=σxσyσzτxyτyzτzxT

σ2d=[σxσyτxy]

Strain vector:

ϵ3d=ϵxϵyϵzγxyγyzγzxT

ϵ2d=[ϵxϵyϵxy]

The constitutive equations to relate strain to stress can be done with Hooke’s law assuming either plane stress or plain strain with matrix [D].

σ=Dϵ

The plane strain relation is defined as the following for the 2 dimensional assumption

σxσyτxy=E1+ν1-2ν1-νν0-ν10001-2ν2ϵxϵyγxy

The Plane stress relation is defined as the following can be used for the 2d assumption

σxσyτxy=E1-v21ν0ν10001-ν2ϵxϵyγxy

## Kinematic Equations, Equilibrium

For a liner-elastic body the total potential energy assuming small strain

Πu=UIu-UEu

Where UI is the internal potential energy (strain energy) and UE is the external potential energy in the form of external forces and u is the displacement field.

The internal potential energy is defined as strain energy which is the work done by the internal stresses while deforming a body from zero strain to its final strain state.

dU=σTdϵ

Need to integrate the strain over the entire volume

U=V0ϵσTdϵ

Stress is related to strain by the constitutive matrix σ=Dϵ

U=V0ϵDϵTdϵ

Integrating with respect to epsilon yields

Uu=12VϵTDϵdV

External Work

Assuming there are no body forces or surface tractions

UEu=iuiTPi

This gives the potential energy statement of:

Πu=12VϵTDϵdV-iuiTPi

By definition the virtual work is equal to zero. Therefore, taking the first variation of the potential energy statement gives

δΠu=0

δΠ=δUI-δUE=VϵTDδϵdV-iδuiTPi=0

Gives

VϵTDδϵdV=iδuiTPi

Relating strains to displacements gives

Where ϵ, u are defined as

u=N{d} Where N is the matrix of shape functions

ϵ=∂u Strain is known to be the partial derivatives of displacement

{d} is the elemental degrees of freedom ie {u1v1u2v2}

Therefore, substituting u into the epsilon gives

ϵ=∂N{d}

Next, the B matrix is defined as the partial derivatives of the shape functions

B=∂[N] therefore substituting into the strain displacement relationship gives

ϵ=B{d}

VBTDBdV{d}=iδuiTPi

Therefore, the stiffness matrix is

Ke=VBTDBdV

For the 2 case the volume spans x, y and the constant in the z for a constant thickness t.

Ke= tyxBTD[B] dx dy

### Deriving the B Matrix

Starting from the strain displacement formula provided previously

ϵ=Bd

Strain is the change in displacement with respect to a given direction

ϵx=∂u∂x,  ϵy=∂v∂y,  γxy=∂u∂y+∂v∂x

Re-arranging in matrix form

εxεyγxy=100000010110∂u∂x∂u∂y∂v∂x∂v∂y

The core logic of the finite element logic is to use shape functions to approximate the global displacements where u, and v are approximated by the shape functions N.

ux, y= i=1nNix, yui,  vx,y= i=1nNix,yvi

uv =Ni00Niuivi

Computing the derivatives

u,xu,yv,xv,y=Ni,x0Ni,y00Ni,x0Ni,yuivi

Substituting into the strain displacement relationship

εxεyγxy=100000010110Ni,x0Ni,y00Ni,x0Ni,yuivi

This simplifies to

εxεyγxy=Ni,x00Ni,yNi,yNi,xuivi

Therefore, the B matrix is where i is each node within the element

Bi=Ni,x00Ni,yNi,yNi,x

## Boundary Conditions

Boundary conditions for all analysis are limited to being a fixed constraint of either x, y or both eliminating translation in these directions. It is assumed that there are no rotations and everything remains planar.

# Stiffness Derivation

## Calculation in the Global System

The previously shown elemental stiffness calculation’s volume can be calculated across x, y for a quadrilateral element. This approach is feasible for perfectly square elements. However, it becomes challenging to determine the proper integration limits when the element becomes distorted. Virtually every element within a FE model has some level distortion. Thus the elements will be mapped from the global coordinate systems into an idealized natural coordinate system as shown in Figure 6.

[Ke]= tyxBTD[B] dx dy

## Calculation in the Element Natural Coordinate system

The natural coordinate system for the quadrilateral element is in the ξ,η system with the 4 corners at (-1, -1), (1, -1), (1, 1), (-1, 1). Regardless of the element’s physical position, rotation, or skew; it will always map to the same four points.

Figure 6: Global to Natural Coordinate System

As shown previously the displacements can be defined as

uv=∑Ni(x,y)ui∑Ni(x,y)vi

However, the shape functions here are defined in the x, y coordinate system. The natural coordinate system with the coordinates  ξ, η demands that the shape functions be functions of ξ, η.

Where N are the shape functions that can be used for a liner quadrilateral elements which are also used in the mapping from the global and natural coordinate systems

N1=141-ξ1-η,	N2=141+ξ1+η

N3=141+ξ1+η,	N4=141-ξ1+η

The problem encountered is that the shape functions are in term so of ξ, η and not u, v. Thus a coordinate transformation must take place.

xiyi= J ξiηi

The Jacobian is defined as:

J =∂x∂ξ∂x∂η∂y∂ξ∂y∂η

And the determinant of the Jacobian is

J=∂x∂ξ∂y∂η-∂x∂η∂y∂ξ

The bounds of the integration can now be written in-terms of ξ, η with the following relation

dxdy=det J dξdη

Therefore

ke(x, y)=t∫BTCBdet⁡[ J] dξdη

Element Strain Displacements are given by

ε=Bd

εxεyγxy=100000010110∂u∂x∂u∂y∂v∂x∂v∂y

To get results in terms of ξ, η the multivariable chain rule is used.

∂u∂x=∂u∂ξ∂ξ∂x+∂u∂η∂η∂x,  ∂u∂y=∂u∂ξ∂ξ∂y+∂uη∂η∂y

∂v∂x=∂v∂ξ∂ξ∂x+∂v∂η∂η∂x,  ∂v∂y=∂v∂ξ∂ξ∂y+∂vη∂η∂y

Re-writing in matrix form

u,xu,yv,xv,y=ξ,xη,x00ξ,yη,y0000ξ,xη,x 00ξ,yη,yu,ξu,ηv,ξv,η

Substituting in the strain displacement relation

εxεyγxy=100000010110ξ,xη,x00ξ,yη,y0000ξ,xη,x 00ξ,yη,yu,ξu,ηv,ξv,η

uξ,η= i=1nNiξ, ηui,  vξ,η= i=1nNiξ,ηvi

uv =Ni00Niuivi

Computing the derivatives

u,ξ u,ηv,ξv,η=Ni,ξ0Ni,η00Ni,ξ0Ni,ηuivi

As the shape functions are functions of ξ, η their derivatives can be taken

Substituting in the final strain displacement relation shows the following where i is the index of nodes

εxεyγxy=100000010110ξ,xη,x00ξ,yη,y0000ξ,xη,x 00ξ,yη,yNi,ξ0Ni,η00Ni,ξ0Ni,ηuivi

With the B Matix being

B=100000010110ξ,xη,x00ξ,yη,y0000ξ,xη,x 00ξ,yη,yNi,ξ0Ni,η00Ni,ξ0Ni,η

This can be simplified into the following.

B=ξ,xη,x0000ξ,yη,yξ,yη,yξ,xη,xNi,ξ0Ni,η00Ni,ξ0Ni,η

B=ξ,xNi,ξ+η,xNi,η 00ξ,yNi,ξ+η,yNi, ηξ,yNi,ξ+η,yNi, ηξ,xNi,ξ+η,xNi,η

εxεyγxy=ξ,xNi,ξ+η,xNi,η 00ξ,yNi,ξ+η,yNi, ηξ,yNi,ξ+η,yNi, ηξ,xNi,ξ+η,xNi,ηuivi

Returning back to the elemental stiffness calculation, all required variables are now known

ke(x, y)=t∫∫BTCBdet⁡[ J] dξdη

## Gauss Quadrature

While programs like python with sympy or Maple can perform symbolic integration, it is inefficient compared to numerical methods. Gause integration says that an integral can be evaluated as a summation of functions evaluated at integration points with weights applied.

-11fxdx= i=11wifxi

For double integration

-11-11fx,ydxdy= i=1nj=1nwiwjfxi, yi

Substituting into the stiffness calculation

ke(x, y)=ti=1nj=1n[wiwjBTCBdet⁡[ J] dξdη]

Insert the table showing the different orders of weights and points

The Gauss Points and Weights shown in Figure 7 are selected by the polynomial order of the element’s shape functions.

Figure 7: Gauss Quadrature Weights and Points [3]

The shape functions for each element are shown below

Table 1: Gauss Points and Weights Per Element Type

| Element | Shape Function (Ex) | Gauss Points | Gauss Weights |
| --- | --- | --- | --- |
| CTRIA3 | 1-ξ-η | 1/3 | 0.5 |
| CQUAD4 | 0.25*(1-ξ)(1-η) | ±13 | 1 |
| CQUAD6 | η*ξ4-η4+ξ22-ξ4-14 | ±13 | 1 |
| CQUAD7 | 0.25*ξ(1-ξ)(η-1) | -35, 0, 35 | 59, 89, 59 |
| CQUAD8 | -14(1-η)(1-ξ)(1+ξ+η) | -35, 0, 35 | 59, 89, 59 |

# Element Shape Functions

## CTRIA3

Some Text Here

| Nodal Point | Shape Function | ∂N∂ξ | ∂N∂η |
| --- | --- | --- | --- |
| N1: (0,0) | 1-ξ-η | -1 | -1 |
| N2: (1, 0) | ξ | 1 | 0 |
| N3: (0, 1) | η | 0 | 1 |

Sone Text Here

## CQUAD4

Some Text Here

| Nodal Point | Shape Function | ∂N∂ξ | ∂N∂η |
| --- | --- | --- | --- |
| N1: (-1, -1) | 14(1-ξ)(1-η) | 0.25η-0.25 | 0.25ξ-0.25 |
| N2: (1, -1) | 14(1+ξ)(1-η) | 0.25-0.25η | -0.25ξ-0.25 |
| N3: (1, 1) | 14(1+ξ)(1+η) | 0.25η+0.25 | 0.25ξ+0.25 |
| N4: (-1, 1) | 14(1-ξ)(1+η) | -0.25η-0.25 | 0.25-0.25ξ |

fdafdas

## CQUAD8

Some Text Here

| Nodal Point | Shape Function | ∂N∂ξ | ∂N∂η |
| --- | --- | --- | --- |
| N1: (-1, -1) | -14(1-η)(1-ξ)(1+ξ+η) | 0.25(-η-2ξ)(η-1) | 0.25(-2η-ξ)(ξ-1) |
| N2: (1, -1) | -14(1-η)(1+ξ)(1-ξ+η) | 0.25(η-1)(η-2ξ) | 0.25(2η-ξ)(ξ+1) |
| N3: (1, 1) | -14(1+η)(1+ξ)(1-ξ-η) | 0.25η+1η+2ξ | 0.25(2η+ξ)(ξ+1) |
| N4: (-1, 1) | -14(1+η)(1-ξ)(1+ξ-η) | 0.25-η+2ξ(η+1) | 0.25(-2η+ξ)(ξ-1) |
| N5: (0, -1) | 12(1-η)(1-ξ)(1+ξ) | ξ(η-1) | 0.5ξ2-0.5 |
| N6: (1, 0) | 12(1-η)(1+ξ)(1+η) | 0.5-0.5η2 | η(-ξ-1) |
| N7: (0, 1) | 12(1+η)(1-ξ)(1+ξ) | ξ(-η-1) | 0.5-0.5ξ2 |
| N8: (-1, 0) | 12(1-η)(1-ξ)(1+η) | 0.5η2-0.5 | η(ξ-1) |

Some text here

## CQUAD7

Some text here

| Nodal Point | Shape Function | ∂N∂ξ | ∂N∂η |
| --- | --- | --- | --- |
| N1: (-1, -1) | 141-ξ1-η -12N5 | 0.25η-0.5ξη-1-0.25 | 0.25ξ(1-ξ) |
| N2: (1, -1) | 141+ξ1-η-12N5-12N6 | 0.25η2-0.25η-0.5ξ(η-1) | 0.5ηξ+1-0.25ξ2-0.25ξ |
| N3: (1, 1) | 141+ξ1+η-12N6-12N7 | 0.25η2+0.25η+0.5ξ(η+1) | 0.5ηξ+1+0.25ξ2+0.25ξ |
| N4: (-1, 1) | 141-ξ1+η-12N7 | -0.25η+0.5ξη+1-0.25 | 0.25ξ(ξ-1) |
| N5: (0, -1) | 12(1-ξ2)(1-η) | ξ(η-1) | 0.5ξ2-0.5 |
| N6: (1, 0) | 121+ξ1-η2 | 0.5-0.5η2 | -ηξ+1 |
| N7: (0, 1) | 12(1-ξ2)(1+η) | -ξ(η+1) | 0.5-0.5ξ2 |

Some text here

## CQUAD6

Some Text Here

| Nodal Point | Shape Function | ∂N∂ξ | ∂N∂η |
| --- | --- | --- | --- |
| N1: (-1, -1) | 141-ξ1-η -12N5 | 0.25η-0.5ξη-1-0.25 | 0.25ξ(1-ξ) |
| N2: (1, -1) | 141+ξ1-η-12N5-12N6 | 0.25η2-0.25η-0.5ξ(η-1) | 0.5ηξ+1-0.25ξ2-0.25ξ |
| N3: (1, 1) | 141+ξ1+η-12N6 | 0.25η(η+1) | 0.5ηξ+1+0.25ξ2+0.25ξ |
| N4: (-1, 1) | 141-ξ1+η | -0.25η-0.25 | 0.25ξ(ξ-1) |
| N5: (0, -1) | 12(1-ξ2)(1-η) | ξ(η-1) | 0.5ξ2-0.5 |
| N6: (1, 0) | 121+ξ1-η2 | 0.5-0.5η2 | -ηξ+1 |

Some Text Here

# Elasticity Solution

The classical elasticity solution for the infinite plate is defined as the following equations

σrr=σ21-a2r2+σ21+3a4r4-4a2r2cos2θ

σθθ=σ21+a2r2-σ21+3a4r4cos2θ

σrθ= -σ21-3a4r4+2a2r2sin2θ

The critical stress occurs when r = a and θ=π2

σrr=σ21-11+σ21+3-4cosπ=0

σθθ=σ21+1-σ21+3cosπ=σ--2σ=3σ

σrθ= -σ21-3 +2sinπ=0

## Femap Solution

Figure 8: Femap Solution for Open Hole Tension Plate

## Python Solver

Fdasfas

Figure 9: Python Solver for Open Hole Plate

fdas

This validates the solver is solving stiffness and stresses appropriately.

# Kt Solution From Initial Problem

# References

| [1] | B. L. S. a. W. J. H. Horn, Failure, Fracture Mechanics, Fatigue, and Damage Tolerance, Wichita State University Department of Aerospace Engineering. |
| --- | --- |
| [2] | W. D. Pilkey, Perterson's Stress Concentration Factors, John Wiley & Sons, Inc. |
| [3] | "UPC.edu," [Online]. Available: https://numfactory.upc.edu/web/Calculo2/P2_Integracio/html/Gauss1D.html. |

# ElementRepository.py

# KeCalc.py